# Start of day - testing setup

## Scan 001

- Quick scan with PI stage (Steps per wavelength = 4)

- Exposure too high (200ms)

## Scan 002

- Exposure reduced (100 ms)

- More steps per wavelength (30)

- Shifted Newport stage +5 μm (probe arrives earlier)

## Scan 003

- Gone for lunch - averaging over 5 acquisitions per step

- Shifted Newport stage -3 μm (probe arrives later)

- CEP dropped out through scan.

- Fibre drifted (beam lock feedback not on)

# Power scan

## Scan 004

- ND: 0.1 → 10-0.1 ~ 80%

- TP (PD):

  - 6.3 mW (wrong settings on PM)

  - 3.8 mW (correct settings on PM, measured after Scan 005, with 207 mW on PP)

- TP (TC): 9 mW

- PP (TC): 216.5 mW

## Scan 005

- ND: 0.2 → 10-0.2 ~ 63%

- TP (PD):

  - 5.95 mW (wrong settings on PM, measured before scan)

  - 5.9 mW (wrong settings on PM, measured after scan)

- TP (TC):

- PP (TC): 203 mW (measured after scan)

- CEP dropping out.

## Scan 006

- ND: 0.2 → 10-0.2 ~ 63%

- TP (PD): 3.2 mW (measured after scan)

- TP (TC):

- PP (TC): 207 mW

- CEP dropping out.

## Scan 007

- ND: 0.3 → 10-0.3 ~ 50%

- TP (PD):

  - 2.8 mW (measured before scan)

  - 3 mW (measured after scan)

- Power meter (PD) not very good (probably saturating, and using incorrect settings). Changed to Coherent thermocouple (TC)

- TP (TC):

  - 3.2 mW (after scan)

  - 5.5 mW (after scan, with lights off)

- PP (TC): 240 mW

## Scan 008

- ND: 0.4 → 10-0.4 ~ 40%

- TP (PD):

- TP (TC):

  - 4.8 mW (before scan)

  - 4.7 mW (after scan)

- PP (TC): 210 mW

## Scan 009

- ND: 0.5 → 10-0.5 ~ 32%

- TP (PD):

- TP (TC):

  - 3.7 mW (before scan)

  - 3.8 mW (after scan)

- PP (TC): 240 mW

## Scan 010

- ND: 0.8 → 10-0.8 ~ 16%

- TP (PD):

- TP (TC):

  - 2.5 mW (before scan)

  - 2.4 mW (after scan)

- PP (TC): 240 mW

- possible problem with CEP

## Scan 011

- CEP unstable - laser tweaked to give better CEP, CE value changed.

- ND: 1.0 → 10-1.0 ~ 10%

- TP (PD):

- TP (TC):

  - 1.5 mW (before scan)

  - 1.2 mW (after scan)

- PP (TC):

  - 240 mW (before scan)

  - 242 mW (after scan)

- Test pulse blocked - oops (can use as reference)

## Scan 012

- ND: 2.0 → 10-2.0 ~ 1%

- TP (PD):

- TP (TC):

  - 0.6 mW

  - 0.4 mW (?)

  - unable to get reliable power measurement

- PP (TC):

- CEP miles off, data from this scan deleted

## Scan 012

- ND: 3.0 → 10-3.0 ~ 0.1%

- TP (PD):

- TP (TC):

- PP (TC):

## Scan 013

- ND: 4.0 → 10-4.0 ~ 0.01%

- TP (PD):

- TP (TC):

- PP (TC):