# CEP variation with PP

- Inserted (2x 105176 Layertec uncoated wedges, 2°) and (1x Thorlabs ND 0.6) into test pulse arm.

- Put spectrometer on leakage of interferometer.

- Tried to replace R=99% BS after 50% recombination BS with a reflective ND2 filter. The absorption of the filter was too high, so the output power was less.

- Increased pressure in fibre and adjusted wedges.

- Fixed CEP of laser:

- Beam was not centred on f-2f iris.

- CEP RMS 140 / 180

- Cut-off appears to depend strongly on CEP and wedge settings.

## Scan001

- Testing zero position of stage

## Scan002

- Moved NP stage to centre pulse better

- CEP tests

## Scan003

- CEP ~ 0 = φ0

## Scan004

- CEP = φ0 + π/2

# CEP Scan of TP 01

- FDI exposure =

- CEP shift in fused silica:

- 2π per 58 μm →  0.415 mm

- CEP shifted tested by moving Agilis stage and observing shift in fringes. A movement of 1000 resulted in ~π phase shift.

- SI fringes taken at ~ NP0 + 0.1 mm (1000 shots)

## Scan005

- Moved wedge to near minimum thickness (Position 45000)

- Finding zero delay

- Moved NP to 4.4665 mm

## Scan006

- Moved NP -20 μm

- Scan aborted after pulse acquired (fringes stopped after ???)

## Scan007

- Moved Agilis stage back and forth - lost position so resetting scan.

- Checking zero position on NP stage.

- Test pulse fairly long

## Scan008

- Finer resolution over limited range

- Took SI fringes after AS scan

- Agilis: 47000

## Scan009

- Moved Agilis stage -100 steps

- Agilis: 46900

- SI fringes before (Scan009.csv) and after (Scan009a.csv) AS scan.

## Scan010

- Agilis: 46800

## Scan011

- Agilis: 46700

## Scan012

- Agilis: 46600

- Scan012a.csv may have been overwritten by Scan013.csv. Scan013 taken again with correct filename.

## Scan013

- Agilis: 46500

- Scan013.csv is correct. Scan012a.csv may not be.

## Scan014

- Agilis: 46400

## Scan015

- Agilis: 46300

## Scan017

- Agilis: 46200

## Scan018

- Agilis: 46100

- Agilis: 46000? Must have made a mistake above somewhere

## Scan019

- Not moving NP stage between measurements.

- Not taking SI fringes with Thorlabs spectrometer

- Agilis: 45900

## Scan020

- Agilis: 45700

## Scan021

- Agilis: 45500

## Scan022

- Agilis: 45300

## Scan023 to ???

- Agilis: 45100

- FDI exposure increased to

- Went for dinner

- Managed 2.5 scans

- 3rd scan CEP not locked upon return

## Scan026

- Moved fixed wedge to thinner side (less glass)

- Moved translation wedge to thinner side

- Scanning full PI range coarsely

- Agilis: 48000

## Scan027

- Moved PI stage 20 μm

- Agilis: 47500

## Scan028

- Agilis: 47000