## Scan 02.01

Computer crashed.

FDI settings:

FDI Setting	Value

x-bin	2

y-bin	4

left	250

right	1200

bottom	540

top	626

Exposure	100ms

Frequency	10MHz

Use eV	Yes

MCP position	20mm

Agilis stage was on ~-6990

Agilis stage not moved and object position reset to -6990

Agilis stage moved (-7000:10:-6000) with relatively fine sampling resolution of PI stage (30nm).

Reconstructed temporal field

[collapse collapsed]

Reconstruction Code

%% Create figure

%  =============================================================================

% Create a figure with default values set

PRINT = false;

PRINT = true;

fig = figure(1);		%	Figure handle (always use figure 1)

clf(fig);				%	Clear current figure

%	Ensure window is not docked

set(fig, 'WindowStyle', 'normal');

drawnow;			%	Ensure figure is undocked before proceeding

%	Set the figure size so that the aspect ratio matches the final output (in

%	this case its 1:1)

set(fig, 'Units', 'pixels', 'Position', [10 50 1000 500]);

%	Set renderer (requires OpenGL for transparency / lighting, painters is good

%	for line art)

set(fig, 'Renderer', 'painters');

%	Set paper position mode:

%	THIS IS IMPORTANT TO ENSURE THE IMAGE IS EXPORTED CORRECTLY

set(fig, 'PaperPositionMode', 'auto');

%	Set default font sizes etc

FN = 'Times';				%	Default font series

FS = 16;					%	Default font size

LW = 2;						%	Default line width

set(fig, ...

'DefaultLineLineWidth', LW, ...

'DefaultTextFontName', FN, ...

'DefaultTextFontSize', FS, ...

'DefaultAxesFontName', FN, ...

'DefaultAxesFontSize', FS, ...

'DefaultAxesLineWidth', LW)

%% Load data

%  =============================================================================

DirName = find_folder({...

'/media/LG_EXT_HDD/data/'});

DirName = fullfile(DirName, 'xuv/attosecond_sampling/2012-12-12');

fprintf('Folder location:\n%s\n\n', DirName);

%	Get filenames

%	----------------------------------------------------------------------------

%	List of files

fnames = dir(fullfile(DirName, '*.mat'));

%	Extract CEP scan 01 files

fnames = regexpi({fnames.name}, 'CEPScan01_\d{6}.mat', 'match');

ind = cellfun(@(str) ~isempty(str), fnames);

fnames = fnames(ind);

% fnames = fnames(6:10);

%	Number of files

Nf = length(fnames);

indq = (8:80)';

Nq = length(indq);

indz = (1:192);

Nz = length(indz);

load(fullfile(DirName, fnames{1}{:}), 'z');

%	Delay axis

Nw1 = 2^14;

tau = 2*z(indz)/.3;

tau = tau - mean(tau);

dtau = mean(diff(tau));

%	Frequency axis

dw = 2*pi/(	Nw1 * dtau);

w = ((0:Nw1-1)'-floor(Nw1/2))*dw;

indw = (w>1.5 & w<5);

w = w(indw);

Nw = length(w);

%	Temporal axis

Nt1 = 2^12;

ddw = 10;

dt = 2*pi/(Nt1*dw*ddw);

t1 = ((0:Nt1-1)'-floor(Nt1/2))*dt;

indt = (abs(t1)<50);

t = t1(indt);

Nt = length(t);

Etr = zeros(Nz, Nf);

Et = zeros(Nt, Nf);

Ets = Et;

Ew = zeros(Nw, Nf);

L = zeros(Nf, 1);

fprintf('Loading data ...\n\t(of %d)', Nf);

for nf=1:Nf

fprintf('\t%d', nf);

%	Load interferogram

load(fullfile(DirName, fnames{nf}{:}), 'I', 'Position');

L(nf) = Position;

%	Normalize data

I0 = mean(I, 2);

I = bsxfun(@minus, I, I0);

I = bsxfun(@rdivide, I, max(I, [], 2));

%	Calculate temporal field

E = GeneralizedAverage(I(indq, indz).', 20, hamming(16));

Etr(:, nf) = E;

%	Calculate spectrum

E = Time2Freq(E, Nw1, 1, [0 1]);

E = E(indw);

Ew(:, nf)  = E;

%	Calculate analytic field

E = E(1:ddw:end);

E = Freq2Time(E, Nt1).*exp(-i*w(1)*t1);

Et(:, nf) = E(indt);

%	Shift data

I = abs(E).^2;

t0 = Weighted_Mean(t1, I.*(IGT1e-7), 1);

Ets(:, nf) = spline(t1-t0, E, t);

% 	E = Freq2Time(E.*exp(i*w*t0));

% 	Ets(:, nf) = E(ind);

% 	Ets(:, nf) = spline(t1-t0, E, t).*exp(-i*w(1)*t);

% 	Ets(:, nf) = circshift(Et(:, nf), floor(Nt/2)+1-find(t0LSt, 1, 'first'));

end

L = (L - mean(L))*2.2794e-3;

fprintf('\n');

%% Plot Data

clf

Nfh = floor(Nf/2)+1;

L1 = .09;

L2 = .51;

B1 = .13;

B2 = .58;

W = .4;

H = .4;

ha = axes('Parent', fig, 'Position', [L1 B2 W H]);

[m, n] = max(Etr(:, Nfh));

imagesc(L, tau-tau(n), Etr/m);

set(gca, 'YLim', [-11 11], 'CLim', [-1.2 1.2], 'XTickLabel', [])

text(-1, -8, '(a) Averaged only', 'FontSize', FS-2);

% title('Reconstructed electric field (averaged only)')

% xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

ind = (1:10:Nf)';

% line(bsxfun(@plus, ones(Nt, 1), L(ind)') + .1*abs(Et(:, ind)/m).^2)

axes('Parent', fig, 'Position', [L2 B2 W H]);

[m, n] = max(real(Et(:, Nfh)));

imagesc(L, t-t(n), real(Et)/m);

set(gca, 'YLim', [-11 11], 'CLim', [-1.2 1.2], 'XTickLabel', [], ...

'YAxisLocation', 'Right')

text(-1, -8, '(b) Fourier filtered', 'FontSize', FS-2);

% title('Reconstructed electric field (Fourier filtered)')

% xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

% line(bsxfun(@plus, .1*abs(Et(:, ind)/m).^2, L(ind)'), t-t(n), 'Parent', ha, ...

% 	'LineStyle', '--');

axes('Parent', fig, 'Position', [L1 B1 W H]);

imagesc(L, t, real(Ets)/m);

set(gca, 'YLim', [-11 11], 'CLim', [-1.2 1.2])

text(-1, -8, '(c) GD compensated', 'FontSize', FS-2);

% title('Reconstructed electric field (GD removed)')

xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

axes('Parent', fig, 'Position', [L2 B1 W H]);

imagesc(L, t, abs(Ets)/m);

set(gca, 'YLim', [-11 11], 'CLim', [0 1.2], 'YAxisLocation', 'Right')

text(-1, -8, '(d) Intensity, GD compensated', 'FontSize', FS-2);

% title('Reconstructed temporal intensity (GD removed)')

xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

if exist('PRINT', 'var') && PRINT

print(fig, 'CEPDependence01.png', '-dpng', '-r600');

end

[/collapse]

(a) Averaged only

I normalized the HHG spectra (as previously described) and performed an average in the cut-off. One can clearly see the group delay dependence.

Taking the refractive index of fused silica to be 1.45 (@800nm), 0.5 microns of fused silica results in a 2pi phase shift - this is confirmed in the image.

(b) Fourier filtered

I performed some Fourier filtering to generate the analytic signal: I Fourier transformed into the spectral domain (with padding) and selected the positive sideband, downsampled and Fourier transformed back (with padding). This way I can easily obtain the intensity, phase or real signal.

This is the same as (a) except with some noise removed due to the filtering.

(c,d) [Intensity] GD compensated

I use the temporal intensity to find the RMS delay and remove this value for each wedge setting.

I've plotted the GD compensated real electric field in (c) and the temporal intensity in (d).

One can clearly see that the temporal intensity and CEP varies little.

Scan 02.03

Reconstruction

[collapse collapsed]

Reconstruction code

%% Create figure

%  =============================================================================

% Create a figure with default values set

PRINT = false;

PRINT = true;

fig = figure(1);		%	Figure handle (always use figure 1)

clf(fig);				%	Clear current figure

%	Ensure window is not docked

set(fig, 'WindowStyle', 'normal');

drawnow;			%	Ensure figure is undocked before proceeding

%	Set the figure size so that the aspect ratio matches the final output (in

%	this case its 1:1)

set(fig, 'Units', 'pixels', 'Position', [10 50 1000 750]);

%	Set renderer (requires OpenGL for transparency / lighting, painters is good

%	for line art)

set(fig, 'Renderer', 'painters');

%	Set paper position mode:

%	THIS IS IMPORTANT TO ENSURE THE IMAGE IS EXPORTED CORRECTLY

set(fig, 'PaperPositionMode', 'auto');

%	Set default font sizes etc

FN = 'Times';				%	Default font series

FS = 16;					%	Default font size

LW = 2;						%	Default line width

set(fig, ...

'DefaultLineLineWidth', LW, ...

'DefaultTextFontName', FN, ...

'DefaultTextFontSize', FS, ...

'DefaultAxesFontName', FN, ...

'DefaultAxesFontSize', FS, ...

'DefaultAxesLineWidth', LW)

%% Load data

%  =============================================================================

DirName = find_folder({...

'/media/LG_EXT_HDD/data/'});

DirName = fullfile(DirName, 'xuv/attosecond_sampling/2012-12-12');

fprintf('Folder location:\n%s\n\n', DirName);

%	Get filenames

%	----------------------------------------------------------------------------

%	List of files

fnames = dir(fullfile(DirName, '*.mat'));

%	Extract CEP scan 01 files

fnames = regexpi({fnames.name}, 'CEPScan03_\d{6}.mat', 'match');

ind = cellfun(@(str) ~isempty(str), fnames);

fnames = fnames(ind);

fnames = fnames(1:15);

%	Number of files

Nf = length(fnames);

indq = (8:80)';

Nq = length(indq);

indz = (4:1263);

Nz = length(indz);

load(fullfile(DirName, fnames{1}{:}), 'z');

%	Delay axis

Nw1 = 2^14;

tau = 2*z(indz)/.3;

tau = tau - mean(tau);

dtau = mean(diff(tau));

%	Frequency axis

dw = 2*pi/(	Nw1 * dtau);

w = ((0:Nw1-1)'-floor(Nw1/2))*dw;

indw = (w>1.5 & w<5);

w = w(indw);

Nw = length(w);

%	Temporal axis

Nt1 = 2^12;

ddw = 5;

dt = 2*pi/(Nt1*dw*ddw);

t1 = ((0:Nt1-1)'-floor(Nt1/2))*dt;

indt = (abs(t1)<75);

t = t1(indt);

Nt = length(t);

Etr = zeros(Nz, Nf);

Et = zeros(Nt1, Nf);

Ets = zeros(Nt, Nf);

Ew = zeros(Nw, Nf);

L = zeros(Nf, 1);

fprintf('Loading data (of %d) ...\n', Nf);

for nf=1:Nf

fprintf('\t%d', nf);

%	Load interferogram

load(fullfile(DirName, fnames{nf}{:}), 'I', 'Position');

L(nf) = Position;

%	Normalize data

I0 = mean(I, 2);

I = bsxfun(@minus, I, I0);

I = bsxfun(@rdivide, I, max(I, [], 2));

%	Calculate temporal field

E = GeneralizedAverage(I(indq, indz).', 20, hamming(70));

Etr(:, nf) = E;

%	Calculate spectrum

E = Time2Freq(E, Nw1, 1, [0 1]);

E = E(indw);

Ew(:, nf)  = E;

%	Calculate analytic field

E = E(1:ddw:end);

E = Freq2Time(E, Nt1).*exp(-i*w(1)*t1);

% 	Et(:, nf) = E(indt);

Et(:, nf) = E;

%	Shift data

I = normalize(abs(E).^2, 1, 'intensity');

% 	t0 = Weighted_Mean(t1, I.*(I>1e-3), 1);

% 	Ets(:, nf) = spline(t1-t0, E, t);

[~, n] = max(I);

E = circshift(E, floor(Nt1/2)+1-n);

Ets(:, nf) = E(indt);

% 	E = Freq2Time(E.*exp(i*w*t0));

% 	Ets(:, nf) = E(ind);

% 	Ets(:, nf) = spline(t1-t0, E, t).*exp(-i*w(1)*t);

end

L = (L - mean(L))*2.2794e-3;

dL = mean(diff(L));

fprintf('\n');

%% Plot Data

clf

dL1 = 50*dL;

L1 = linspace(L(1), L(end), dL1).';

Nfh = floor(Nf/2)+1;

left1 = .09;

left2 = .51;

bottom1 = .1;

bottom2 = .4;

bottom3 = .7;

width = .4;

height = .28;

ha = axes('Parent', fig, 'Position', [left1 bottom3 width height]);

[m, n] = max(Etr(:, Nfh));

imagesc(L, tau-tau(n), Etr/m);

set(gca, 'YLim', [-100 100], 'CLim', [-1.2 1.2], 'XTickLabel', [])

text(-20, -80, '(a) Averaged only', 'FontSize', FS-2);

% title('Reconstructed electric field (averaged only)')

% xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

ind = (1:10:Nf)';

% line(bsxfun(@plus, ones(Nt, 1), L(ind)') + .1*abs(Et(:, ind)/m).^2)

axes('Parent', fig, 'Position', [left2 bottom3 width height]);

[m, n] = max(real(Et(:, Nfh)));

imagesc(L, t1-t1(n), real(Et)/m);

set(gca, 'YLim', [-100 100], 'CLim', [-1.2 1.2], 'XTickLabel', [], ...

'YAxisLocation', 'Right')

text(-20, -80, '(b) Fourier filtered', 'FontSize', FS-2);

% title('Reconstructed electric field (Fourier filtered)')

% xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

% line(bsxfun(@plus, .1*abs(Et(:, ind)/m).^2, L(ind)'), t-t(n), 'Parent', ha, ...

% 	'LineStyle', '--');

axes('Parent', fig, 'Position', [left1 bottom2 width height]);

imagesc(L, t, real(Ets)/m);

set(gca, 'YLim', [-11 11], 'CLim', [-1.2 1.2], 'XTickLabel', [])

text(-20, -8, '(c) GD compensated', 'FontSize', FS-2);

% title('Reconstructed electric field (GD removed)')

% xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

I = normalize(abs(Ets).^2, 1, 'intensity');

[m, n] = max(max(I));

axes('Parent', fig, 'Position', [left2 bottom2 width height]);

imagesc(L, t, I/m);

set(gca, 'YLim', [-11 11], 'CLim', [0 1.2], 'YAxisLocation', 'Right', ...

'XTickLabel', [])

text(-20, -8, '(d) Intensity, GD compensated', 'FontSize', FS-2, 'Color', 'y');

% title('Reconstructed temporal intensity (GD removed)')

xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

E = normalize(Ets, 1);

E = E.*(abs(E)>1e-2);

% clf; imagesc(abs(E))

% return

E = interp2(L.', t, E, L1.', t, 'spline');

[m, n] = max(max(real(E)));

axes('Parent', fig, 'Position', [left1 bottom1 width height]);

imagesc(L1, t, real(E)/m);

set(gca, 'YLim', [-11 11], 'CLim', [-1.2 1.2])

text(-20, -8, '(e) GD compensated, interpolated', 'FontSize', FS-2);

% title('Reconstructed electric field (GD removed)')

xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

axes('Parent', fig, 'Position', [left2 bottom1 width height]);

imagesc(L1, t, abs(E/m).^2);

set(gca, 'YLim', [-11 11], 'CLim', [0 1.2], 'YAxisLocation', 'Right')

text(-20, -8, '(f) Intensity, GD compensated, interpolated', ...

'FontSize', FS-2, 'Color', 'y');

% title('Reconstructed temporal intensity (GD removed)')

xlabel('Additional fused silica [\mum]')

ylabel('Time [fs]');

grid on

if exist('PRINT', 'var') && PRINT

print(fig, 'CEPDependence03.png', '-dpng', '-r600');

end

[/collapse]

The good news is that the data shows a CEP dependence.

The bad news is that the data's not great. This is probably because the laser became unstable in the middle - one can clearly see this in the raw data.

CEP should shift \(2\pi\) per \(52\mu\)m of fused silica (calculation from Tobi). It appears the CEP shifts faster than this (about \(2\pi\) per \(10\mu\)m).

The wedge axis is not 100% accurate because the amount the stage moves each step varies, and I used an average from some calibration data (here). However, taking the refractive index of 1.45, assuming little dispersion, the total group delay across that axis is about 230fs, which corresponds to the maximum travel of the stage, so I believe those numbers.