# Objective

- Measure spectra of single and both arms to perform spectral interferometry (SI) and measure probe pulse (PP) temporal distribution.

# Procedure

- Power from HCF: 0.810 W

- Agilis (wedge) stage centred by hand

- Agilis stage set to position: -7000

# Attosecond Sampling

- NP0: 4.535 + 0.006

## Data set 1

- CEP unstable

- spectrum with large peak around 700 nm

- CoarseScan000001.mat

- CoarseScanCentred000001.mat

- FineScan_dz20um000001.mat

## Data set 2

- CEP locking and HCF alignment adjusted

- HCF wedge position changed: (2.0717 → 3.7999)

- Initial pinhole aperture as previous day

- After tweaking, more modulation than data 1 scans

- CoarseScan000002.mat

- FineScan_dz10um000002.mat

## Data set 3

- pinhole closed to approx ⅔ of previous diameter

- δz = 50nm

- CoarsePH2_3_000001.mat

- Pinhole closed to approx ½.

- δzcoarse = 50nm

- δzfine = 10nm

- CoarsePH1_2_000001.mat

- FinePH1_2_000001.mat

# SI measurements:

## Leakage measurements

- NP stage set to: 4.630 mm

- period doubles across full spectrum (shorter at lower wavelengths)

- position chosen to get fine fringes without loosing spectral modulation depth (watching on leakage) ~ 10 points per fringe @ 575 nm (~ lowest wavelegth usable to see fringes)

- 1000 interferograms

- leakage_fringes_exp00035ms.csv

## Direct measurements

- measuring spectra before entering chamber using fibre and foam cube

- PP & TP single arms recorded (1000 scans), both with no saturation and to check low intensity spectral bits

- PP_exp00007ms.csv

- PP_exp00016ms.csv

- TP_exp00200ms.csv

- TP_exp00375ms.csv

- Scan wedge stage position from -15000 to +5000 (step = 1000)

- 100 scans recorded for each stage position

- exposure adjusted to maintain signal peak around 80% - 90% of maximum

- ~ 10/15 points per fringe @ 575 nm

- wm????? = wedge position in steps (m = minus, p = plus)

- SI_wm14000.csv

- SI_wp03000.csv

# Results

## Wedge dispersion calibration

- The agilis stage introduces 2.2794 nm / step of fused silica

### Method 1

- The unmodulated harmonic spectrum, \(I_0\), is calculated by averaging in the wings (where there is little or no modulation)

- The unmodulated harmonic spectrum is subtracted from the modulated spectrum for each delay

- The subtracted modulated spectrum is normalized to the peak value for each harmonic order to give the measured TP field.

- The spectrum is calculated by Fourier transforming the field with padding, and then normalized to peak intensity (within the appropriate range).

#### Matlab code

% Axes

Nw1 = 2^14;

tau = 2*z/C;

dtau = mean(diff(tau));

dw = 2*pi/(Nw1*dtau);

w = ((0:Nw1-1)'-floor(Nw1/2))*dw;

indw = (w>2.12 & w<3.355);

w = w(indw);

Nw = length(indw);

% Raw harmonic spectrum

ind = (1:100)';

I0 = GeneralizedAverage(I(:, ind), 20, hamming(10));

% Retrieved field

E = bsxfun(@minus, I, I0);

E = bsxfun(@rdivide, E, max(E, [], 2));

% Retrieved spectrum

I = abs(Time2Freq(E.', Nw1, 1, [0 1])).^2;

I = I(indw, :);

### FinePH1_2000001.mat

- Performed analysis using both the raw HHG intensity and the HHG intensity after interpolation onto a linear frequency axis. Found that there was little difference in the results. Below are the results from the raw HHG intensity only.

- Figure below shows the retrieved spectrum as a function of the harmonic frequency. This is calculated via the absolute magntiude square of the Fourier transform along the delay axis, and then normalized to the peak intensity for each harmonic frequency. The unmodulated HHG is overlaid in green along the top. The independently measured non-intensity calibrated spectrum is overlaid in black on the right.

The figure below shows (left) the reconstructed temporal electric field and temporal intensity (inset); and (right) reconstructed spectral intensity (blue) and phase (green) and independently measure spectrum (black).

### Method 2

- The position of the 'cut-off' is calculated as a function of delay.

- This can be approximated by finding the contours of the HHG spectrum. This gave very similar results to method 1. However, there is too much overlap between the lower frequency cut-off and higher frequency harmonics.

- It is found that the general shape of the HHG spectrum in the 'cut-off' does indeed shift in frequency as expected. However, the intensity also changes, making an automatic quantitative reconstruction difficult.

## Other analysis

- The TP was reconstructed via the attosecond sampling.

- The PP phase was calculated by subtracting the retrieved SI phase from the TP and combining with the square root of the independently measured spectrum from the non-intensity calibrated Thorlabs spectrometer.

- The harmonic spectrum was calculated using the SFA model for Argon, and the peak intensity modified until the cut-off matched the measured spectrum. The peak intensity was estimated to be I0 ~ 1.93x1014 Wcm-2

- Given the input energy of ~180 μJ,  I estimate the Gaussian focal spot to have an e-2 radius of w0 ~ 80 μm