## Comments

- Mounted diagnostics wedge on servo motor to automatically switch between diagnostics and HHG lines.

## Finding Zero Delay

### FindZeroDelay001.mat

- NP stage scanned from 6.1 to 5.9mm.

- Found fringes near 6.05mm

### FindZeroDelay002.mat

- NP stage scanned from 6.11 to 6.05mm

- Zero delay found at 6.082mm

### FindZeroDelay000001.mat

- Scan of PI stage.

- NP0 = 6.08mm

- Full range of PI stage

### FindZeroDelay000002.mat

- Scan of PI stage:

- NP0 = 6.1

- Full range of PI stage

### FindZeroDelay000003.mat

- ???

### FindZeroDelay0000004.mat

- PI scan with fine resolution, limited range

## Repeatability measurements

- Performed 21 scans to see if data is consistent

## Comments

- Found that wedge on servo was unstable (metal plate is too thin). Removed servo and placed wedge back on flip mount - stability much improved

- Camera 1 not quite in focus - camera moved to as close to focus of PP as possible

## Re-test

- Fibre compression wedges scanned a couple of times to find best compression and CEP setting

- Optimal position = 12.8mm

- Coarse scan: FindZeroDelay003.mat

- PI scan: FindZeroDelay000005.mat

- PI Scan: FindZeroDelay000006.mat with finer resolution - saw good 'streaking' of harmonic emission.

## Repeatibility Measurements

- Same settings repeated 5 times  Repeatibility_01_%06d.mat

- SI measurements taken immediately after with same settings. (1hr later)