# Measurements performed:

- Dispersion of TP (data no good)

- Repeatability of a measurement (data no good)

- Dependence on TP intensity (3 intensity values, data OK)

- SI between TP & PP (data good, but little or no decent corresponding HHG data)

# Summary

- Measurements FindZeroDelayxxx for xxx = 01-06 before optimization.

- HHG beamline was reoptimized

## TP Optimization

- Added 300um of fused silica to TP arm

- Performed SI with 2 different wedge positions to calibrate additional dispersion in TP.

- Hopefully FTL TP occurs with Ag=10mm.

## PP CEP scan

- Optimization of HHG signal after tweaking

- Time ~ 15:30

## Test scans

- Ag = 10

- Found zero delay (using human fringe analysis).

- Test01 = Coarse scan over full range - Dispersion_Coarse ??

- Test02 = Fine scan with averaging (5 measurements) - Dispersion_Fine ??

- CEP tweaked.

- Test03 = Not so fine scan, no averaging.

- Test04 = Reduced iris slightly.

- Test05 = Reduced iris some more

# Results

## TP Intensity Dependence

## Repeatability Measurements

- Fine scan at previous settings repeated 10 times.

## Dispersion Scan

- NP stage moved +.05mm for SI measurements

- Same settings as before, except Ag moved.

- Fibre / CPA optimized

- Interferometer realigned

- Harmonics Optimized

- Ag moved +3mm each measurement

- NP moved -0.035mm each measurement

- SI performed before (SI_Dispxx.csv) and after (SI_Dispxxa.csv) modulation scan.

- Coarse scan (Dispersion_Coarse_xxx.mat) performed before each scan to find optimal range to scan PI stage over.

- Fine scan (Dispersion_Fine_xxx.mat) contains actual data with averaging (over 5 images).

- Performed an additional measurement at Ag=10 to match up with previous data.

- Did not move PI stage to 19 on SI_..._10a.

Data not very good - CEP dropped out in-between and beam drifted such that harmonics changed.

Wedge calibration

Glass insertion ~ 52um of fused silica per 1mm of stage travel.

Group delay ~ 260fs per 1mm of stage travel