Solutions:

- Buffer 1 – 80mM NaCl, 0.1% BSA, in HEPES pH 7.2

- Buffer 2 – 20mM NaCl, in HEPES pH 7.2

- Buffer 3 – 20mM NaCl, 0.1% BSA, in HEPES pH 7.2

- Buffer 4 –HEPES pH 7.2

- 0.1% Polylysine in milliQ

- diluted anti-FliC (black dot)

- Check/record OD of cells = 0.45 – 0.65 no compromise.  Grown in sux from frozen stocks (see other protocol)

- Clean coverslips in flow hood to dry

- Turn on laser (if 632nm) and PC in microscope room

- Take 10µl of IgG stock beads (red label) + 1.5 ml Buffer 1 and spin down (13k, 7min) & remove by pipette

- Resuspend finally in 20µl (or 50 µl) Buffer 1

- Truncate filaments by shearing 40 times in “shearing device”.

- Spin down ~0.6ml (7k, 3min)

- Resuspend (pipette up and down with P200) in 300ul Buffer 4 (can use #2 but hard to avoid clumps)

- Make a standard “tunnel” flow-cell with double-sided tape

- Inject flow-cell with 0.1% polylysine. Incubate for 20sec.

- Flush through 100µl Buffer 2 – VERY IMPORTANT otherwise clumpy cells.

- Wick through 20µl cell suspension and leave upside down in humidity chamber (5-10 min).  Check for stuck cells on microscope.

- Make 1/5 FliC dilution

- Put beads on to sonicate for 15min

- Start all PC programs etc

- Flush through 2 x 50µl Buffer 3

- Wick through 10µl of 1/5 dilution anti-flagellin rabbit IgG in Buffer 3 and incubate 5-15min (inverted)

- Wick through 20µl of α-IgG bead suspension and incubate 5 min (inverted)

- Check using bench top microscope that you can see a reasonable number of spinning beads. If not try incubating longer.  If too many clumps then put beads on to sonicate for longer and wash out slide with BAB.

- Gently flush through 50µl Buffer 2 and then 2x50µl HEPES

Flow cell NOTES:

- need 4x20µl bead tubes per flow cell

- 130ul 1/10 anti-FliC

- Flush way more than 4 drops after polylysine

- After bead injection, use syringe from other direction so less volume required