Dot Blot Protocol

- Cut PVDF membrane 23cm x 17.5cm

- Place on plastic tray with the 10 x 9 dot pattern on over the light box (no methanol req)

- Spot 0.5, 1, 2 or 5ul of sample/standard (exactly the same sample incl dye as you would load for a western) as many times as will fit on the blot

- Leave on light box for 10-30mins until each spot has completely dried

- Wash for 1hr min (I’ve always left it overnight) in 5% marvel

- Proceed with western protocol as normal

Note:

Initially when working out approx conc’s of Ab and standards it is useful to load different amounts and or dilutions

Analysis

- Scan in blot as colour TIFF (or multi-tiff)

- Open Photoshop, convert to greyscale and flip/invert if necessary, save changes

- Open ImageQuant5.1 from Elaine’s PC (WREN, only one that works!), open Tiff

- Volume report setup, tick Name, Vol, Average, Area only

- Select circles (hold shift), and get select standards in order

- Volume report, open excel file and copy and paste over numbers

- Then move circles to empty space and get values for backgrounds, take away from standards to give corrected values

- Plot standard curve and remove crap dots, then get y INTERCEPT, and SLOPE; use formulas, do not take off graph regression line – sometimes this is wrong!  I tend to get a linear relationship which makes the analysis easy.

- Repeat for all samples and then open ‘26a. Estimating number of copies of protein per cell’ document to do the calculations.