Photographing Rs swarm plates

Swarms can be shot without Casamino acids but 0.05% works well (375ul of 10% stock per 75ml plate).  The problem is the relative proportions of each strain change which is less than ideal!  So you can either present the rubbish accurate photos without Casamino or manipulate the Casamino plates so that you get the correct proportions eg by photographing certain strains early/late etc.

Camera setup:

- Place plate on black card, illuminated with light box at 45 deg using a stool

- Canon 20D on my tripod with bottom legs only fully extended and with the rod the camera is attached to v bottom

- Camera 18-55mm lens, fully zoomed (55mm)

- On ‘P’ setting, M JPEG record

Analysis:

- Photoshop > view> actual pixels

- Select appropriate fixed box size and copy to new file

- Auto contrast

- Save for web