Supplementary information - The charging mechanism of Li2MnO3

Niccoló Guerrini‽, Liyu Jin‽, Juan G. Lozano†, Kun Luo†, Adam Sobkowiak, Kazuki Tsuruta‡, Felix Massel§, Laurent-Claudius Duda§, Matthew R. Roberts‽ and Peter G. Bruce*,†.

†Departments of Materials and Chemistry, University of Oxford, Parks Road, Oxford OX1 3PH, United Kingdom

‡Japan Synchrotron Radiation Research Institute (JASRI), 1-1-1 Kouto, Sayo, Hyogo 679-5198, Japan

§Department of Physics and Astronomy, Division of Molecular and Condensed Matter Physics, Uppsala University, Box 516, S-751 20 Uppsala, Sweden

″Etteplan Sweden AB, SE-171-54 Solna, Sweden

‽ Dyson Technology Ltd., Tetbury hill, Malmesbury, SN16 0RP, United Kingdom

PXRD

Figure S1 Li2MnO3 powder X-ray diffraction pattern collected using Cu-radiation. Tick marks indicate allowed reflections for the monoclinic C2/m space group of Li2MnO3.

SEM

Figure S2 SEM images for Li2MnO3 at two different magnifications. The morphology consists of primary particles (50 to 150 nm) agglomerated into 20-50 micron secondary particles.

TGA-MS

The experiment was performed on a ~20 mg sample contained in an alumina crucible under inert atmosphere of Ar. The temperature was increased with a rate of 10 °C/min between room temperature and 1000 °C.

The TGA (cyan line, Fig S4) registered a 2% mass loss for the sample whilst the evolution of O2 and CO2 was detected by the mass spectrometer. Even if all the mass loss is accounted for by decomposition of Li2CO3 from the material surface, it can be easily calculated that the resulting amount of carbonate would account for less than 10% of the extracted capacity in the galvanostatic cycling. As well oxygen could be released from the material lattice as a consequence of Manganese reduction from 4+ to 3+ oxidation state at high temperature.

Figure S4 TGA-MS analysis performed on pristine Li2MnO3.

Contribution of Li2CO3 to the charge and gas loss

From the TG-MS experiment a 2% weight loss is detected. If all the loss is attributed to Li2CO3 present along with Li2MnO3 in the pristine material, following the equation for Li2CO3 electrochemical oxidation reported in the main text, From Faraday’s law it is calculated that 14% of the charge passed in the electrochemical experiment is associated with Li2CO3 decomposition of which 2/3 leads to CO2 formation and 1/3 to O2.

Combining the information attained from TG-MS and OEMS it is possible to evaluate the amount of charge extracted as gas-loss that is associated respectively with Li2MnO3 and Li2CO3

| Charging Regime | Total charge passed (mAhg-1) | Charge passed minus charge associated with Li2CO3 oxidation (mAhg-1) | Charge from O2 (mAhg-1) | Charge from O2 corrected for Li2CO3 contribution (mAhg-1) | Charge from CO2 (mAhg-1) | Charge from CO2 corrected for Li2CO3 contribution (mAhg-1) | Total Charge from gas loss (mAhg-1) | Total Charge from gas loss corrected for Li2CO3 contribution (mAhg-1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Galvanostatic | 102 | 87.5 | 44.5 | 39.7 | 55 | 45.3 | 99.5 | 85 |
| GITT | 145 | 130.5 | ~5 | ~0.2 | 142 | 132.3 | 144 | 129.5 |

Hence, we get the following table.

Calculation:

For 1 gram of pristine material is evaluated that 98% is Li2MnO3 and 2% Li2CO3, hence 0.98g and 0.02g respectively. Employing Faraday’s law Q = nF/3.6MW it is possible to calculate the theoretical capacity of Li2CO3 (725.43 mAhg-1). Hence for 0.02g are attained 14.5 mAh, corresponding to the charge contribution of Li2CO3 for 1 g of pristine material. Of these 14.5 mAh, 2/3 are associate with CO2 evolution and 1/3 with O2.