# REACH Dhaka Observatory

# Methodology for Water Quality Data

Dr. Mohammed Abed Hossain

Ms. Shammi Shawal

Nafis Sadik Khan

Contents

Introduction	1

Sampling points and selection	1

Rationale of Site Selection	5

Upper Turag (Kaliakoir to Konabari)—(Point 47 & 48)	5

Upper Turag (Konabari to Rustampur)—(Point 1 -7)	5

Lower Turag (Rustampur to Amin bazar)— (Point 8 to 10)	6

Buriganga River (Point 11-19)	6

Tongi Khal and Pagar Canal (Point 20-25 & 55)	7

Balu River and Canals (Point 26 -30, 56-58)	8

Bangshi-Savar River (Point 31-32)	9

Dhaleswari River (Point 33 -40)	9

Shitalakhya (Point 41 -46 and 52)	10

Bangshi River (Kaliakoir to Nayarhat)—Point 49 to 51	11

Kaliganga River (Point 53-54)	11

Sample Collection, Preservation and Analysis	12

Field Measurement of Water Quality Parameters	12

Laboratory Measurement of Water Quality Parameters	12

References	15

# Introduction

The data for Dhaka rivers water quality has been derived from periodical field measurement of water quality parameters and laboratory measurement of key parameters using water samples collected from field visits.

The river system and canals covered are:

- Turag River

- Buriganga River

- Tongi Khal

- Balu River

- Dhaleswari  River

- Shitalakhya River

- Bangshi River

- Bangshi-Savar River (also known as Karanatali River)

- Kaliganga

- Pagar Canal

- Begunbari Canal

- Bashundhara Canal

# Sampling points and selection

The work on deciding sampling point followed a number of reconnaissance visits, GIS mapping exercise and satellite image analysis. The coverage also increased with time. At first Upper Turag (From Konabari to Rustampur), Tongi Khal and Balu river reach were included. JICA funded SATREPS project experience was used for Tongi Khal and Balu in 2017. At first the number of points was comparatively higher. With observance of data the number of points were reduced and also altered. Pollution understanding and catchment wide pollution model development resulted in extension of the monitoring coverage. We followed a stepwise manner consistent with our expanding modelling capacity and coverage. The monitoring and modelling targeted the areas undergoing most dynamic industrialization and socio-economic changes and severe environmental challenges. We modified our sampling plan to cover more areas in line with areas of interest of the GoB with regard to Greater Dhaka, e.g., Tanner shifting from Hazaribagh beside Buriganga to Savar (Harindhara-Hemayetpur) beside Dhaleswari, growing economic zones at Dhaleswari and Shitalakhya etc.

The lower part of Turag and Buriganga river were included in regular monitoring in 2018, and Bangshi-Savar in late 2018. The sampling coverage further expanded to include Dhaleswari and Shitalakhya in 2019, Bangshi, Kaliganga, Pagar canal, Begunbari canal and Bashundhara canal in 2020. Finally, the monitoring covered a total of 58 sampling points, collected monthly up to February 2020, and then seasonally till July 2021. Locations are listed in table 1 and illustrated in map 1.

Map 1: REACH water quality monitoring points

Table 1: Sampling point locations for Water Quality Monitoring for Greater Dhaka

| Point | Location Name | River system | Latitude | Longitude |
| --- | --- | --- | --- | --- |
| 1 | u/s Konabari | Upper Turag (Konabari to Rustampur) | 24.01945 | 90.34443 |
| 2 | Powerplant | Upper Turag (Konabari to Rustampur) | 23.9948 | 90.34186 |
| 3 | d/s Konabari | Upper Turag (Konabari to Rustampur) | 23.98681 | 90.32572 |
| 4 | d/s Kashimpur | Upper Turag (Konabari to Rustampur) | 23.98042 | 90.33772 |
| 5 | Yearpur | Upper Turag (Konabari to Rustampur) | 23.92568 | 90.34812 |
| 6 | Ashulia | Upper Turag (Konabari to Rustampur) | 23.89519 | 90.33463 |
| 7 | Rustampur | Upper Turag (Konabari to Rustampur) | 23.8795 | 90.3503 |
| 8 | Birulia Bridge | Lower-Turag | 23.85146 | 90.33932 |
| 9 | Botanical Garden | Lower-Turag | 23.81745 | 90.3405 |
| 10 | Mirpur | Lower-Turag | 23.78946 | 90.33922 |
| 11 | Mirpur Bridge | Buriganga | 23.78292 | 90.33538 |
| 12 | Hazaribag | Buriganga | 23.73496 | 90.35441 |
| 13 | Kamrangir Char | Buriganga | 23.71671 | 90.35992 |
| 14 | Chandni Ghat | Buriganga | 23.71022 | 90.39367 |
| 15 | Sadar Ghat | Buriganga | 23.70409 | 90.40971 |
| 16 | DholaiKhal | Buriganga | 23.69661 | 90.41836 |
| 17 | BCF Bridge | Buriganga | 23.68716 | 90.4278 |
| 18 | Pagla | Buriganga | 23.6648 | 90.45238 |
| 19 | Zazira | Buriganga | 23.63233 | 90.46342 |
| 20 | Jamaldia | Tongi Khal | 23.89441 | 90.36478 |
| 21 | Ijtema Field, Tongi | Tongi Khal | 23.89172 | 90.38989 |
| 22 | Tongi rail bridge | Tongi Khal | 23.88179 | 90.40636 |
| 23 | Hossain Dyeing Ltd. | Tongi Khal | 23.88925 | 90.42315 |
| 24 | Mausaid | Tongi Khal | 23.89918 | 90.43655 |
| 25 | Uttara ABM city | Tongi Khal | 23.88724 | 90.45995 |
| 26 | u/s Purbachal | Balu | 23.86218 | 90.47498 |
| 27 | d/s Purbachal | Balu | 23.82718 | 90.4873 |
| 28 | Beraid | Balu | 23.7777 | 90.4779 |
| 29 | d/s Begunbari Khal | Balu | 23.75971 | 90.48347 |
| 30 | Demra | Balu | 23.72791 | 90.50095 |
| 31 | d/s of Turag Branch Outfall | Bangshi-Savar | 23.79974 | 90.30303 |
| 32 | Amin Bazar Landfill | Bangshi-Savar | 23.79691 | 90.32647 |
| 33 | Nayarhat Bridge | Dhaleswari | 23.91102 | 90.22923 |
| 34 | Bhagalpur Bridge | Dhaleswari | 23.79757 | 90.24703 |
| 35 | Kadamtoli Bridge | Dhaleswari | 23.75366 | 90.24965 |
| 36 | Ruhitpur Bridge | Dhaleswari | 23.66822 | 90.28876 |
| 37 | d/s of BSCIC | Dhaleswari | 23.66107 | 90.31206 |
| 38 | d/s of Dhaleswari Bridge | Dhaleswari | 23.6357 | 90.36069 |
| 39 | Baluchar Bridge | Dhaleswari | 23.61136 | 90.42233 |
| 40 | Mukterpur Ghat | Dhaleswari | 23.56632 | 90.51359 |
| 41 | Kanchan Bridge | Shitalakhya | 23.83562 | 90.54478 |
| 42 | ACI Salt | Shitalakhya | 23.79051 | 90.52276 |
| 43 | Demra Ghat | Shitalakhya | 23.71733 | 90.50334 |
| 44 | u/s of Adamjee EPZ | Shitalakhya | 23.66487 | 90.52252 |
| 45 | u/s of Taanbazar Ghat | Shitalakhya | 23.6131 | 90.5062 |
| 46 | Kalagachia | Shitalakhya | 23.57237 | 90.52548 |
| 47 | Kaliakoir Bridge | Upper Turag (Kaliakoir to Konabari) | 24.08281 | 90.20916 |
| 48 | Boroibari Bridge-Madhyapara | Upper Turag (Kaliakoir to Konabari) | 24.09963 | 90.26697 |
| 49 | d/s of Kaliakair Road | Bangshi | 24.06962 | 90.21446 |
| 50 | d/s of DEPZ Canal | Bangshi | 23.94241 | 90.23062 |
| 51 | Hazipur Nayarhat | Bangshi | 23.92053 | 90.22616 |
| 52 | d/s Ghorashal FC | Shitalakhya | 23.989 | 90.63628 |
| 53 | d/s Sholla Bridge | Kaliganga | 23.71932 | 90.1872 |
| 54 | d/s Paragram Kaliganga Bridge | Kaliganga | 23.7129 | 90.23926 |
| 55 | Pagar Canal (to Tongi) | Tongi Khal | 23.90066 | 90.43258 |
| 56 | Bashundhara Canal (to Balu) | Balu | 23.80976 | 90.47701 |
| 57 | Begunbari Canal (to Balu) | Balu | 23.76039 | 90.47974 |
| 58 | Daripara Bridge, Pubail | Balu | 23.93208 | 90.49082 |

# Rationale of Site Selection

## Upper Turag (Kaliakoir to Konabari)—(Point 47 & 48)

The Upper Turag (Kaliakoir to Konabari) River has mostly rural settlements and agricultural setup with some brick kilns. There are several industries including agro, spinning, apparel, polymer, pharmaceutical etc. and two industrial parks upstream of this reach, pollution load from which flows through this river.

• Point 47: This point is located near Kaliakoir Bridge at Turag (near the offtake of Bangshi). This is the starting point for Turag River within Greater Dhaka and therefore a necessary point for watershed scale pollution assessment. It will help to assess the pollution load coming from the industrial parks.

• Point 48: This point is located at Boroibari Bridge, Madhyapara. Surrounding areas are agrarian. This one is located to cover for areas between Konabari and Kaliakoir. This will help assess pollution contribution of Kaliakoir and movement of pollution plume.

## Upper Turag (Konabari to Rustampur)—(Point 1 -7)

The Upper Turag River (from Kaliakoir to Rustampur) is rapidly industrialized and contributing large pollution load unforeseen in the early nineties when these areas were agrarian and had considerable Sal Forest cover. The river reach has land-use ranging from industrialized/municipal to rural. Based on the findings from previous sampling and analysis, seven points are being sampled from February 2018 for this river reach. The rationale for the sampling point choices is given below:

• Point 1: Surroundings of this point have mostly agrarian setup. Some brickfields are also located at the right bank of the river. This point is also located u/s of Konabari industrial zone, which will help assess pollution contribution of the industrial area.

• Point 2: Located at d/s of Kodda Bridge. Both upstream and downstream parts on the right bank are surrounded by brick fields. Kodda power plant and Summit Gazipur power plant is situated near this point. This point will help determine pollution coming from the power plants.

• Point 3: Located at d/s of Konabari. Many industries are situated at the right bank of the point while the left bank is surrounded by agricultural lands. This point will assess the pollution load coming from Konabari industrial area.

• Point 4: Located a little downstream of Kashimpur area as it is rapidly transforming into industrialized area as well as municipal development,

• Point 5: Located at Yearpur where the residential development is likely to produce new townships within 2030. The area is now in a rural setting. It also marks the beginning of the Ashulia Lake that works as a retention basin (gradually decreasing in coverage now due to filling, encroachments, and brick fields).

• Point 6: Located downstream of Ashulia Bridge. The surroundings of the left bank of this point have an agricultural setup. On the other hand, suburban Ashulia town is situated at the right bank of the point. This point location will help to assess pollution contribution from municipal area.

• Point 7: Located at Rustampur, a little upstream of the confluence with Tongi Khal. It works as a bifurcation point during monsoon and low tide while it works as a confluence of Tongi Khal and Upper Turag to form flow of Lower Turag.

## Lower Turag (Rustampur to Amin bazar)— (Point 8 to 10)

The reach from Rustampur to Amin Bazar is hydrologically referred to as the Lower Turag and referred to as just Turag in most cases especially with regard to navigation especially circular river route of Dhaka. Data at Rustampur collected previously indicated large variation of pollution between monsoon and dry season.

• Point 8: Located d/s of Birulia Bridge and the offtake of the Tongi. The left bank of the river has urban setups while the right bank has agricultural setups.

• Point 9: Located near the botanical garden. One side of the river has an agrarian setup and another side has an urban setup with some recreational spots near the bank.

• Point 10: Located at Mirpur, u/s of the confluence with Buriganga and Bangshi-Savar. This will allow for upstream pollution tracking as well as municipal contribution to pollution at Lower Turag.

## Buriganga River (Point 11-19)

Buriganga river has nine sampling points from February 2018. Out of the 9 points, 8 (point 11-18) are regular sampling points usually monitored by DoE. The sampling plan included one Buriganga additional sampling point near the confluence of Buriganga and Dhaleswari so that the point marks the end point for Buriganga discharging onto the Dhaleswari.

• Point 11: Located downstream of Mirpur/Aminbazar Bridge near Aminbazar Landing Station which marks the beginning of the Buriganga River. Many brick fields are placed downstream of this point.

• Point 12: Located d/s of Hazaribag canal which used to drain the old tannery site in Dhaka at Hazaribagh. This point would help to assess the impact of old tannery town and monitor recovery.

• Point 13: Located downstream of Kamrangir Char Canal near the launch terminal. Both sides of the river are covered with densely urban setups.

• Point 14: Located at Chandni Ghat which used to be an intake point for a small water treatment plant of WASA and also an important launch terminal. Monitoring this point would help to observe river water quality recovery and also safeguard people and infrastructure from high pollution.

• Point 15: Located at Sadarghat, the largest inland passenger launch terminal in Dhaka and a pollution hotspot.

• Point 16: Located d/s of Dholaikhal Pumphouse which is a major outfall of stormwater from Old Dhaka area.

• Point 17: Located at little d/s of the BCF bridge over Buriganga connecting Dhaka with Mawa. It is a pollution hotspot.

• Point 18: Located downstream of the only sewerage treatment plant of Dhaka and also is subject to loads of industrial and municipal waste.

• Point 19: The point is named the Zazira point, a little u/s of the confluence with the Dhaleswari and close to the Mill Barak Station of BWDB on the Buriganga. Many brick fields are located downstream of this point up to confluence with Dhaleswari.

## Tongi Khal and Pagar Canal (Point 20-25 & 55)

There are 6 sampling points in Tongi Khal. Point 20-25 are the recommended points for monitoring by DoE which includes the modified or recommended changed locations of DoE points.

• Point 20: The first point in Tongi Khal that marks the beginning of the industrial area. It is suitable for assessing how much pollutants from u/s actually contribute. Considering the tidal and two-way flow, it may be suitable for assessing contribution to Turag.

• Point 21: The start of the Ijtema Field where the 2nd largest congregation of Muslims takes place. It is useful for assessing and monitoring pollution load reaching Ijtema Ground.

• Point 22: Located d/s of the Ijtema field and a little downstream of the Tongi Railway Bridge which is considered the most heavily polluted point in the municipal area. It is at the start of the BSCIC area (Govt. industrial area that started back in sixties). It is useful for assessing Ijtema impact as well.

• Point 23: Located at the end of the Partial Embankment of the Tongi Khal on the right bank. The reason for selecting this point is to assess, midway through the BSCIC area, the pollution status and also to assess the pollution level of the water reaching the floodplains on the right bank and polluting the soil and crops.

• Point 24: Located at Mausaid, a little d/s of Pagar where the canal bringing BSCIC area waste meets the Tongi Khal. The point at Mausaid would be suitable for tracking the pollution plume to assess exposure to residential settlements and agricultural lands during flooding.

• Point 25: The last point of Tongi Khal and located before the confluence with Balu. The surroundings of this point are agricultural areas. This point helps to assess pollution load to Balu from Tongi Khal.

• Point 55: The sampling point at Pagar canal. The canal falls into Tongi Khal, between point 23 and 24, just u/s of the ‘Point 24’. Pagar canal is a pollution hotspot carrying BSCIC area waste. The point is about 420 m u/s of the confluence with Tongi Khal. This location is affected by flood and during monsoon polluted water goes to the floodplain.

## Balu River and Canals (Point 26 -30, 56-58)

Balu River is highly contaminated but unfortunately out of the monitoring network of DoE. The REACH project and JICA funded project at BUET have found elevated levels of ammonia and other contaminants indicating the ecosystem have been damaged. The river has high variation in land use at present but with a pattern of urbanization throughout the reach including Purbachal area. There is 1 monitoring station in the upper reach (u/s of confluence with Tongi Khal) and 5 monitoring stations d/s of the confluence.

• Point 58: This point is located upstream of the confluence with Tongi Khal, at the downstream of Daripara Bridge, Pubail and has rural settlements in the banks. This point assesses the water quality of Balu River before joining with Tongi khal.

• Point 26: Industrial effluent, domestic sewage, urban runoff from Tongi area contributes pollution to this area. This point is located just upstream of Purbachal New Town which helps to assess pollution reaching Purbachal area.

• Point 27: Located downstream from the Purbachal (including STPs) at Isapura Bridge. It is suitable for monitoring pollution from Purbachal.

• Point 28: Located d/s of Beraid which is in rural setting but likely to become municipal within next decade. Bashundhara canal falls into Balu river about 5km u/s of this point.

• Point 29: Located about 230m d/s from the point where the Begunbari Canal meets the Balu River. Begunbari Canal is connected to the Hatirjheel area that receives waste from Tejgaon industrial area within central Dhaka. It is connected to Badda, Rampura and Khilgaon areas where a number of pumphouses of WASA would be operational in near future draining stormwater. It would also drain Banani and Gulshan area once Gulshan Lake and Banani Lake gets connected with Hatirjheel area.

• Point 30: This is the monitoring point on the Balu located at Demra before the confluence with Shitalakhya that helps assess pollution load to Shitalakhya from Balu. It is also useful for pollution load assessment.

• Point 56: Sampling point for the Basundhara canal which is located on the southern side of the Basundhara residential area. This canal falls into Balu river between point 27’ and 28, almost 5 km u/s of point 28. It will assess the pollution load of Basundhara residential area. New landfilling activities can be seen on both sides of the canal. The monitoring point is about 800m u/s of the confluence with Balu.

• Point 57: Located at Bengunbari canal which falls into Balu River between point 28 and 29, just u/s of ‘point 29’. It is about 390m u/s from the confluence with Balu. The Begunbari canal is of utmost importance for the Dhaka Watershed as a major part of waste matter from eastern Dhaka and north-eastern Dhaka finds its way to Balu River through this channel. The Hatirjheel lake and the Kamalapur Canal are connected to this channel. Kamalapur Pump House of WASA is discharging into this channel. This point is d/s of Dasherkandi WWTP (waste-water treatment plant). This point will allow for monitoring the impact of Dasherkandi treatment plant on improving the waste load reduction.

## Bangshi-Savar River (Point 31-32)

Bangshi-Savar River originates from Dhaleswari near Savar Bank Town and falls into Turag  near Aminbazar. This river is also known as Karnatali River. Upstream of its outfall is Turag and downstream is Buriganga. This river is getting narrower day by day due to encroachments and being polluted by surrounding industrial and municipal wastes. Water quality is monitored at two points by the REACH project in this river.

• Point 31: is just near the Amin Bazar landfill which receives around 2,000 tons of waste daily (Al Amin, M., 2019). Although planned as a sanitary landfill, the conditions have deteriorated over the years so that the site is now functioning as an open dump. This point would help determine pollution coming from the landfill.

• Point 32: Located about 2km upstream of the outfall to Lower Turag near its confluence with Buriganga. There are many brickfields located on both side of the river between point 31 and 32. This location is important for assessing pollution load of Dhaleswari to Buriganga and vice versa considering the tidal action.

## Dhaleswari River (Point 33 -40)

Dhaleswari River originates from Jamuna, meets with Bangshi River at Nayarhat, Buriganga River at Fatullah and then the merged flow meets with Shitalakhya River d/s of Munshiganj Launch Terminal. Bangshi, Kaliganga and Ichamati River fall into Dhaleswari. Bangshi-Savar is a distributary of this river which also works as tributary during tidal action. CETP of Savar Tannery Industrial Estate discharges into Dhaleswari and it has become the major concern for the health of this river which flows into Meghna.

• Point 33: This is the first point of Dhaleswari and located at Nayarhat Bridge on Dhaka-Aricha Highway, d/s of the confluence with Bangshi, and d/s of Dhaka EPZ. The Dhaka EPZ is located at the floodplain of Bangshi. The waste reaches this point of Dhaleswari through Bangshi. Monitoring water quality is essential to assess pollution load to Dhaleswari system. The BWDB hydrologic station is also located here. This point would help to assess the pollution plume from Dhaka EPZ and Bangshi that is entering into Dhaleswari system affecting Savar town and other places.

• Point 34: Located at Harindhara at Hemayetpur, Savar. DoE has sampling point here. The recommended point for this one is at Bhagalpur Bridge about 1 km u/s of the new leather town at Harindhara and about 3km d/s of the offtake of Bangshi-Savar.

• Point 35: This point is located d/s of Savar Leather City. Reconnaissance visits faced many challenges in fixing the location and finally, Kadamtoli Bridge about 1.5 km downstream from the leather city seemed the best place. This point is of utmost importance as it would catch the pollution load discharging into the Dhaleswari river and its impact on the overall water quality. From this point the river is not navigable throughout the year up to the confluence with Kaliganga. During dry period due to eutrophication hyacinth grow abundantly which makes it quite impossible to move along the river.

• Point 36: Located downstream of the Rohitpur Bridge. It is the first point of Dhaleswari after meeting with Kaliganga. Surroundings of this point have rural settlements. Dhaka Southern Power Generations (55MW) is situated at the upstream of this point. This point is also at the upstream of BSCIC industrial area and will help to assess the water quality before the industrial zone and the pollution coming from the power plant. There is reverse flow to Kaliganga during tidal action and this point will assess the pollution load reaching Kaliganga during that time.

• Point 37: The location is about 500m d/s of BSCIC area in Keraniganj. This is suitable for tracking pollution from the BSCIC area and also a number of industrial zones currently planned.

• Point 38: Located at the downstream of the Dhaleswari Bridge 1. Surrounding areas are agricultural and there are also some new resorts growing up. The floodplain downstream of this point is covered with many brick fields.

• Point 39: Located at the d/s of Baluchar Bridge at Munshiganj. This is about 4 km u/s from the confluence with Buriganga and the last point before the confluence. Downstream of this point both banks are covered with brick fields up to confluence with Shitalakhya. This would be suitable for assessment of pollution load from Dhaleswari alone.

• Point 40: This point is part of DoE’s existing monitoring network. This is located at Muktarpur Ghat, about 300m d/s from the Muktarpur Bridge site. It would serve the purpose of monitoring pollution from industries at Munshiganj. This would be Dhaleswari last monitoring point on the Dhaleswari and can be considered as a kind of exit point of pollutants through Greater Dhaka System to the Meghna.

## Shitalakhya (Point 41 -46 and 52)

Shitalakhya River originates from the Old Brahmaputra, flows east of Narayanganj town, and meets with Dhaleswari near Kalagachhia and the combined flow falls into Meghna. At present, a number of heavy industries including the Adamjee Jute Mills, stand on the banks of Shitalakhya. There are three thermal powerhouses located at Palash, north of Ghorashal, and one at Shiddirganj, on the bank of the river. The important riverport of Narayanganj is also situated on its bank. The river goes under tidal effect for about five months of the year but never overflows its banks. Balu River falls into Shitalakhya at Demra. There are 3 points u/s of this confluence, and 4 points d/s. DoE currently has three monitoring points in this river.

• Point 52: This is the first monitoring point (geographically, considering river course) in this river, located d/s of the Ghorashal Fertilizer Company and the Ghorashal Power Plant at Palash, Narshingdi. This point would help assess the pollution coming from these.

• Point 41: This point is located at Kanchan Bridge on the eastern side of Purbachal New Town. Several jute mills are placed at the left bank of this point. It will be important for monitoring Purbachal area waste load to Shitalakhya, and together with the station at Demra (point 43) can be of use to get industrial and municipal contribution within the reach.

• Point 42: The location is named as d/s of ACI factory near the Murapara Ferry Ghat. It is part of DoE’s existing network and has good connectivity. It is a pollution hotspot.

• Point 43: Located at d/s of Demra Sultana Kamal Bridge and about 700 m d/s of the confluence point with Balu. This point is part of DoE’s existing network. Vegetable oil industries and paper mills are located at the left bank of this point. The point is used as a water intake point for Syedabad Water Treatment Plant. It is a pollution hotspot.

• Point 44: Located about 1 km d/s of Adamjee EPZ to track pollution load from the EPZ. This station will help assess pollution from large industrial units between Demra and Adamjee including powerplants. The entire reach of Shitalakhya is urbanized on both banks. There are cement factories, an oil refinery, and pharmaceutical industries around this point on the right bank and a dockyard on the left bank.

• Point 45: This point would be located downstream of the BIWTA ghat at Narayanganj. It is located at Taanbazar Ghat. It would allow for pollution contribution between Adamjee and Narayanganj city corporation where both industrial pollution load and waste load from urban settlements need to be monitored.

• Point 46: Located at Kalagachia a little distance u/s from the  confluence with Dhaleswari. Both sides of the river are surrounded by many industries like cement industries, agro industries, food processing industries, etc. This is the last point of Shitalakhya river and serves the purpose of monitoring pollution from many industries of Narayanganj district reaching the Meghna via Dhaleswari.

## Bangshi River (Kaliakoir to Nayarhat)—Point 49 to 51

The Bangshi River from which Turag originates houses the Kaliakoir industrial area. This river is not navigable for most of the year except when swelled by the rains of the monsoon.

• Point 49: The point recommended here is downstream of ‘Kaliakoir Industrial Area’ at RHD Kaliakoir Bridge. It is close to the BWDB gage station and suitable for assessing pollution from Kaliakoir industrial cluster. This point is about 1 km d/s of the offtake of Bangshi from Turag.

•Point 50: Located at Bangshi River near Kakran. The river has rural settlements and is currently unaffected by encroachments. One canal joined the river upstream of this point which is connected to DEPZ. During the dry season navigation is a major problem in this river. Due to eutrophication hyacinths grow abundantly which makes it difficult to move along the river.

•Point 51: Located about 1km upstream from Nayarhat. Upstream of this point has rural setups and downstream of this point has urban setups. Navigation is also a problem during the dry season due to hyacinths.

## Kaliganga River (Point 53-54)

Kaliganga River is a meandering river which originates from Dhaleswari River near Saturia, Manikganj and falls into Dhaleswari near Hazratpur, Keraniganj. Water flow increases significantly during the monsoon period and floods the left bank. There are two water quality monitoring points in this river.

• Point 53: is located at d/s from Sholla Bridge and surroundings are agricultural fields and some rural settlements. This point assesses the water quality of Kaliganga which is still unaffected by industrial pollution.

• Point 54: Located at the d/s of Paragram Kaliganga Bridge. This point is about 2.5 km upstream of the confluence with Dhaleswari. It has rural settlements in the surrounding area. This point would be useful to understand the effect of Kaliganga on the Dhaleswari River system in terms of both water quality and flow.

# Sample Collection, Preservation and Analysis

Water samples were collected as grab samples, using an autosampler (Global Water WS755 Sampler) to collect samples from a depth of 2 ft (0.6m) from the water surface. Samples were stored in pre-acid washed plastic sample bottles which are filled in a way to avoid any trapped air above, and then kept inside laboratory-grade ice boxes. The sampler was rinsed with the sample water several times prior to final sampling to equilibrate the surface with the sample water and reduce cross-contamination. In some cases, a manual sampler is also used maintaining the above protocols of collection and storage.

## Field Measurement of Water Quality Parameters

Some water quality parameters were measured in the field, immersing the probes 2 ft below the surface layer. The in-situ measured parameters are listed in Table 2.

Table 2: Field parameters with instrumentation used and accuracy

| Field parameters | Instruments used | Remarks |
| --- | --- | --- |
| GPS location (lat-long) | Garmin Montana 650 | Accuracy 1-3 meters, with proximity alert for 100 meters |
| Temperature | With HACH HQ40D probe | Accuracy 0.1 °C |
| pH | pH probe with HACH HQ40D | Calibrated prior to field visits. Accuracy 0.01 pH units |
| Electrical Conductivity (EC) | EC probe with HACH HQ40D | Calibrated prior to field visits, accuracy 0.1 µS/m |
| Total Dissolved Solids (TDS) | EC probe with HACH HQ40D | Estimated values by HQ40D Reporting to 1 decimal places in mg/L |
| Dissolved Oxygen (DO) | Luminescence DO probe with HACH HQ40D | Checked calibration prior to field visits. Accuracy to 0.1 mg/L |
| Oxidation Reduction Potential (ORP) | ORP probe with HACH HQ40D / HACH handheld ORP probe | Calibrated prior to field visits. Accuracy 0.1 mV |
| Turbidity | VELP turbidimeter | Calibrated prior to field visits. Accuracy 1 NTU |

The turbidity analysis is done with the turbidimeter using part of the sampled water.

## Laboratory Measurement of Water Quality Parameters

Water samples were analyzed in the laboratory for various organic and inorganic parameters and metals. The parameters varied to some degree and not all parameters were covered for all field visits. The organic and inorganic parameters analyzed in laboratories are listed below in Table 3.

Table 3: Organic and inorganic parameters with instrumentation used

| Parameters | Instruments used | Protocols followed and remarks | Limits |
| --- | --- | --- | --- |
| Total Suspended Solids | HACH DR-6000 UV-Vis Spectrophotometer (190-1100 nm range) | HACH pre-calibrated instrument procedure. Adapted from (Krawczyk and Gonglewski, 1959). | 5-750 mg/L |
| Color |  | Using absorbance at 465 nm on filtered samples. Adapted from APHA Method 2120-C (APHA, 2023). | 15-500 mg/L Pt-Co unit |
| Ammonia-N |  | Following the HACH procedure – Nessler method. Adapted from APHA Method 4500-NH3 B & C (APHA, 2023). | 0.02 to 2.50 mg/L NH3-N |
| Nitrate |  | Following the HACH procedure – cadmium reduction method. Adapted from APHA Method 4500-NO3 B (APHA, 2023). | 0.3 to 30 mg/L as NO3- |
| Phosphate (Orthophosphate) |  | Following the HACH procedure – molybdovanadate method. Adapted from APHA Method 4500-P C (APHA, 2023). | 0.3 to 45.0 mg/L PO43- |
| Sulfate |  | Following the HACH procedure. Compatible with USEPA method EPA 375.4 USEPA, 1992). | 2 to 70 mg/L |
| Sulfide |  | Following the HACH procedure – methylene blue method. Compatible with USEPA method EPA 376.2 (USEPA, 1992). | 0.005-0.80 mg/L |
| Alkalinity | HACH DR 16900 digital titrator (accurate to 0.00125 mL) | Titrimetric method, adapted from USEPA Method EPA 310.1 (USEPA, 1983). | 10- 4000 mg/L as CaCO3 |
| Chloride |  | Titrimetric method using AgNO3 solution. Adapted from standard EPA method 9253 (USEPA, 1983). | 10 to 10000 mg/L as Cl- |
| Total Organic Carbon | Sievers InnovOx (GE) Laboratory TOC analyzer | Used supercritical oxidation method. Adapted from USEPA method EPA 415.2 (USEPA, 1983). | 0-1000 mg/L |
| Total Coliform and E. Coli | HACH Coli-Blue24 broth and incubator | Followed standard methods protocols adapted from USEPA method 1604 | N/A |

All chemical reagents used in this study were of analytical grade and ultra-pure water (i.e., milli-Q water with 18.2 MΩ resistance) was used to prepare the necessary standards and for any dilution action performed. Water samples were centrifuged and then filtrated using 0.22 micron membrane filter prior to laboratory analysis, except for suspended solids, Total Coliform (TC), E. Coli, and metals analysis.

Samples were sent to Oxford Laboratory for metals analysis for the period of August 2020-Febryary 2021 (33 metals tested) and BUET Laboratory tested the metals during March 2019 to February 2020 (16 metals). For analysis of metals, all the samples were acid digested. Although acid digestion is not always performed on river water samples in literature, high pollution of the river warranted adoption of protocol for wastewater analysis. Acid digestion of water was performed following EPA Method 3005A using unfiltered water samples (USEPA, 1992). The final digested samples were filtered using 0.22 µm nylon membrane filter.

The digested samples were analyzed using an inductively coupled plasma-optical emission spectrometer (Perkin Elmer Avio 220 Max ICP-OES). Eight standards and a blank were run before starting sample analysis to generate the standard calibration curve for each metal. Two different sets of standards were prepared to account for the matrix effect of digested samples using EPA 3050A. Sixteen metals were analyzed using this method, and the detection limits for those metals are listed in Table 4.

Table 4: Optical emission spectroscopy detection limits and wavelengths used (at BUET)

| Metals | Detection Limits (µg/L) | Wavelength (nm) | Metals | Detection Limits (µg/L) | Wavelength (nm) |
| --- | --- | --- | --- | --- | --- |
| Arsenic (As) | 1.00 | 193.696 | Manganese (Mn) | 0.026 | 257.610 |
| Calcium (Ca) | 0.03 | 317.933 | Molybdenum (Mo) | 0.30 | 202.031 |
| Cadmium (Cd) | 0.10 | 228.802 | Nickel (Ni) | 0.40 | 231.604 |
| Cobalt (Co) | 0.14 | 228.616 | Lead (Pb) | 1.30 | 220.353 |
| Chromium (Cr) | 0.17 | 267.716 | Antimony (Sb) | 1.7 | 206.836 |
| Copper (Cu) | 0.26 | 327.393 | Selenium (Se) | 1.8 | 196.026 |
| Iron (Fe) | 0.08 | 238.204 | Titanium (Ti) | 0.04 | 334.940 |
| Magnesium (Mg) | 0.008 | 285.213 | Zinc (Zn) | 0.07 | 206.200 |

# References

Al Amin, M. (2019, August 29).’ A landfill in flood flow zone.’ Dhaka Tribune. https://www.dhakatribune.com/bangladesh/dhaka/2019/08/29/a-landfill-in-flood-flow-zone

APHA. (2023). Water Environment Federation. Lipps WC, Braun-Howland EB, Baxter TE, eds. Standard Methods for the Examination of Water and Wastewater, 24.

Krawczyk, D., and Gonglewski, N. (1959). Determining suspended solids using a spectrophotometer. Sewage and Industrial Wastes, 31(10), 1159-1164.

USEPA. (1983). Methods for chemical analysis of water and wastes. Methods for chemical analysis of water and wastes, United States Environmental Protection Agency (USEPA), Washington, DC, USA, 1983. pp. 350-351.

USEPA. (1992). Test Methods for Evaluating Solid Waste: Physical and Chemical Methods (SW-846), United States Environmental Protection Agency (USEPA), 1992, Washington, DC, USA.

USEPA. (2002). Method 1604: Total coliforms and Escherichia coli in water by membrane filtration using a simultaneous detection technique (MI medium), United States Environmental Protection Agency (USEPA), 2002, Washington, DC, USA.